# Homebrew Computer RedBoard6809

By Favard Laurent, 2003/2013, Hobby project around 8 bits processor

Updated: 2013, May 24

Summary

Homebrew Computer RedBoard6809	1

Overview	3

History	3

Apple Mac Computer	3

Hardware	4

CPU Board #1	4

I/O Board #2	4

CPU Board schematic	5

I/O Board schematic	6

Memory map	7

Problem and troubleshooting	8

Wires and clock checking	8

First program test:	8

Second test program	9

Last tests programs	9

Software: EK6809Monitor	10

Boot sequence and initialization	11

Monitor header	12

RAM system variables	12

Monitor functions table	12

ROM Expansion at $8000	13

Automatic Execution	13

Applications Expansion	13

Annexe: Oscilloscope screenshots	15

E clock screenshot	15

/CSROM waveform showing regular processor access	15

4060 Q4 signal: Crystal 2.457600 MHz / 16= 153600 Hz	15

Annexe: Source examples	16

Skeleton of Application Expansion ROM	16

Skeleton of Automatic Expansion ROM	17

Skeleton of Diagnostic Expansion ROM	18

Annex: Monitor Screenshots	23

# Overview

The RedBoard 6809 homebrew computer is a small system Motorola 6809 8bits processor based. The system is made-up of two boards, the CPU board which hosts the processor, RAM, ROM, memory decoder and power supply regulation; the I/O board which hosts a 6821 PIA, a 6850 ACIA and clock generation for bauds speed.

RedBoard it’s because PCBs are in red color.

# History

These boards have been designed in 2003 years but then put aside, this project has been picked up in 2012. Because, I’m worked on a small 6809 Emulator as hobby under Mac OSX with Xcode, It was interesting to emulate these boards in the Emulator instead of any other more complicated 6809 based computer. When the Emulator started to works, I wrote a small Monitor in 6809 assembly for fun…Then, It became clear that the final step was to put this one in a real Eprom and run it on a real hardware. The loop was looped!

The CPU was working for the first time on February 1, 2013 and the full computer was alive on Februray 24, 2013.

# Apple Mac Computer

I used a USB/Serial RS-232 adapter PL20xx chip based. Under OSX Lion, you need the following driver: PL2303_Serial-USB_on_OSX_Lion.pkg

To have a terminal program, you can use the built-in Unix command “screen /dev/tty.PL2203 9600” or any other tools like Zterm.

Beside the 6809 emulator running under OSX, I adapted a 6809 assembler from C sources to compile under Xcode .

# Hardware

## CPU Board #1

The goal of this board is to host the full computer system without any I/O. This board is made-up of:

- Simple power supply regulator 7805 based

- Processor 68B09 with external crystal at 8MHz (For a bus frequency at 2MHz)

- Static RAM 62256, 32kB.

- EEPROM Atmel 28C64, 8kB.

- Memory decoder 74HCT138 (3 to 8)

## I/O Board #2

This board allows the CPU to access to a peripheral parallel and serial interface.

- PIA 68B21 for general purpose usage

- ACIA 68B50 for RS-232 communication with a Max232

- Clock generator 4060 with crystal 2.4576 Mhz based for ACIA speed rate

## CPU Board schematic

## I/O Board schematic

## Memory map

For the hardware point of view, the memory map is as follow:

$FFFF

EEPROM for Boot/Monitor	8kB

$E000

$DFFF

I/O 6821, 6850 (Board#2]	4kB

$D000

$CFFF

No hardware in this area	4kB

$C000

$BFFF

No hardware in this area	4kB

$B000

$AFFF

No hardware in this area	4kB

$A000

$9FFF

No hardware in this area	4kB

$9000

$8FFF

No hardware in this area*	4kB

$8000

$7FFF

Static RAM 			32kB

$0000

*Monitor considers this area as the start of ROM expansion. Check the Monitor chapter to more information. However, for the hardware there is nothing specific.

## Problem and troubleshooting

### Wires and clock checking

Check that all wires are correctly done, not any unexpected wires between wrong signals. Then, with a scope, check the E signal clock on the processor (Or Q). The signal must be correct (See screenshot in Annexes)

E = crystal frequency / 4.

### First program test:

We are lucky, 6809 has a great and useful instruction: Sync.

I used a small program (EK6809Boot1.bin) which contains a sync instruction. When the processor executes a sync instruction it stops its activity and wait for an external synchronization, i.e., an interrupt! In this case BA signal = 1 and BS signal = 0. I suggest having two LEDs to visualize the BA/BS status.

So, to check that processor is able to read the ROM, find the correct Reset vector and fetch some instruction before to stop, I burned an EEPROM with the following code:

org		$E000

BootCode:		lds		#$100

ldu		#$100

loop:		sync		;	BA = 1 and BS = 0

bra		loop

Vector:           	rti

marque:		fcc "LAURENT BOOTCODE TEST #1, 20130201"

;	----------------------------------------------------------------------------------------------------

spaceto 	$FFF0				; special LFD directive: fill from last PC = * to here

org	$FFF0

Vectors:		fdb	Vector

fdb	Vector

fdb	Vector

fdb	Vector

fdb	Vector

fdb	Vector

fdb	Vector

fdb	$E000

Troubleshooting:	 I lived issues for a while until having BA/BS corrects. A7 address bus bit was tied to the VCC Power and A1 was not correctly connected to the RAM and ROM chips.

### Second test program

I used EK6809Boot2.bin to check if the I/O board was ok. This program send a character on the serial RS-232 and the binary values %10101010 and %01010101 in order to have something that can be seeing with a scope.

Troubleshooting:	When I did it, it doesn’t work on the RS-232 but was Ok on the PIA. The problem was D1 data was not connected correctly between the both board and signal E was in short-circuit with D1 on the I/O board. This is why the program doesn’t check the status register before to send a character.

### Last tests programs

When previous issues were solved, I tried EK6809Boot3.bin to check that reading status register was ok. Then, I burned EK6809Boot4.bin which adds a simple RAM memory test.

At the end when seems to be ok, I burned the EK6809Monitor.bin and I had the great pleasure to see the monitor started exactly in the same way in the Emulator under OSX.

# Software: EK6809Monitor

The monitor contents a minimal hardware initialization, memory checking and a set of commands in a small CLI. In addition, for fun, I added the tinyBasic in the same EPROM. Burn the EK6809Monitor.bin in a 8kB EEPROM 28C64.

TinyBasic is (C) Copyright 1977 by JOHN BYRNS

Screenshot of the Monitor started

Screenshot of the tiny basic started

## Boot sequence and initialization

The monitor starts at $E000 and immediately executes a bra to bypass the Monitor header:

org		$E000

bra		OSStart

fcc		"6809"			processor code

fcb		1,0			major,minor

fcb		20,12,01,16		date in BCD (YY,YY,MM,DD)

fdb		FunctionsTable		monitor functions table address

OSStart:		Boot code start here

- Initialize system stack pointer to temporary value

- Initialize user stack pointer to temporary value

- Mask all interrupts (IRQ and FIRQ)

- Copy in RAM the addresses of all interrupts vectors

- Reset the ACIA

- Set the ACIA 9600 baud, 8 bits, no parity, 1 stop

- Check for Diagnostic Cartridge  if [$8000] = ‘D’ and [$8001] = ‘G’

  - If yes, load X with a return address

  - Jump at the address stored at $8002

- Enable the interrupts (IRQ and FIRQ)

- Check the memory from $0000 and compute the size

- Store the RAM size at $0000

- Store the end of RAM at $0002

- Set the System stack point to the end of RAM

- Set the User stack point to the below the system stack area

- Set PIA port A and B in input mode

- Check for Automatic Cartridge if [$8000] = ‘A’ and [$8001] = ‘T’

  - If yes, load X with a return address

  - Jump at the address stored at $8002

- Check for Applications “cartridge” code if [$8000] = ‘A’ and [$8001] = ‘P’

  - If yes, through the list of descriptors and display each program available

- Enter in the main CLI loop for await any user command

## Monitor header

$E000 	bra		OSStart

$E002 	fcc		"6809"

$E006 	fcb		1,0			Major, minor

$E008 	fcb		20,13,02,01		BCD date YY,YY,MM,DD

$E00C 	fdb		FunctionsTable		Monitor functions table

## RAM system variables

$0000	RamSize	Size of the RAM

$0002	RamTop	Top RAM address (last address)

$0004	Swi3Vector	Vector address to SW3

$0006	Swi2Vector	Vector address to SW2

$0008	FirqVector	Vector address to FIRQ

$000A	IrqVector	Vector address to IRQ

$000C	SwiVector	Vector address to SWI

$000E	NmiVector	Vector address to NMI

## Monitor functions table

FunctionsTable	+ 0	fdb		PutChar

FunctionsTable	+ 2	fdb		PutHexChar

FunctionsTable	+ 4	fdb		GetChar

FunctionsTable	+ 6	fdb		GetCharUntil

FunctionsTable	+ 8	fdb		WriteHexByte

FunctionsTable	+ 10	fdb		WriteBinByte

FunctionsTable	+ 12	fdb		WriteString

FunctionsTable	+ 14	fdb		ReadString

FunctionsTable	+ 16	fdb		ReadHexFromString

FunctionsTable	+ 18	fdb		$0000				end of table

## ROM Expansion at $8000

The monitor considers the $8000 area as a possible expansion. For that it will check the both address $8000 and $8001 for a magic number. If nothing is found, nothing it’s done. The hardware set only this area to a size to 4kB in accordance to the 74HCT138 memory decoder. If more space is required, the hardware must be updated to change the default memory map decoding as done for example for the Monitor EEPROM at $E000 where two 74HCT138 outputs are combined via NAND gates.

The area isn’t exclusively an EEPROM, but can be a ROM with any additional hardware…

Diagnostic Cartridge

The monitor executes an automatic code with a JMP instruction. Monitor stores in X register a return address if the program executed wants to return to the monitor.

+0		+1

$8000	=	 ‘D’		‘G’

$8002	=	First 6809 instruction

Go back to Monitor: 	jmp	0,x

### Automatic Execution

The monitor executes an automatic code with a JMP instruction. Monitor stores in X register a return address if the program executed wants to return to the monitor.

+0		+1

$8000	=	 ‘A’		‘T’

$8002	=	First 6809 instruction

Go back to Monitor: 	jmp	0,x

### Applications Expansion

The monitor via the RUN command will perform a JSR sub-routine call. So a program must finish with a RTS to return to the Monitor.

+0		+1

$8000	=	‘A’		‘P’

$8002	=	First application descriptor

Descriptor format:

Descriptor1 + CA_Next		2 bytes, address of the next descriptor or NULL if the last

Descriptor1 + CA_Run		2 bytes, address of the program entry (first 6809 instruction)

Descriptor1 + CA_Init		2 bytes, address of the init code (first 6809 instruction)

Descriptor1 + CA_Date		2 bytes, GEMDOS format: DDDDDDDM.MMMDDDDD

Descriptor1 + CA_Time		2 bytes, GEMDOS format: HHHHHMMM.MMMSSSSS

Descriptor1 + CA_Name	C string NULL terminated program name (Ended with ‘\0’).

The code pointed by CA_Run and CA_Init must be terminated by a RTS instruction. CA_Init can be Null.

See in Annexes a source example.

# Annexe: Oscilloscope screenshots

## E clock screenshot

## /CSROM waveform showing regular processor access

## 4060 Q4 signal: Crystal 2.457600 MHz / 16= 153600 Hz

# Annexe: Source examples

## Skeleton of Application Expansion ROM

RomCartidgeStart		equ		$8000

MonitorStart		equ		$E000

;	----------------------------------------------------------------------------------------------------

;	Offsets in ROM header

CPUCode			equ		2

Version			equ		6

Date			equ		8

OffTableRoutines		equ		12

;	----------------------------------------------------------------------------------------------------

;	Offsets of subroutines in functions's Monitor

PutChar			equ		0

PutHexChar		equ		PutChar+2

GetChar			equ		PutHexChar+2

GetCharUntil		equ		GetChar+2

WriteHexByte		equ		GetCharUntil+2

WriteBinByte		equ		WriteHexByte+2

WriteString		equ		WriteBinByte+2

ReadString			equ		WriteString+2

ReadHexFromString		equ		ReadString+2

;	----------------------------------------------------------------------------------------------------

org		RomCartidgeStart

fcc		"AP"			; Applications Cartridge Header

;	----------------------------------------------------------------------------------------------------

CA_Next00:		fdb		CA_Next01

fdb		CARun00

fdb		$0000

fdb		%0100001001110110		;	2013/03/22

fdb		%0111100000000000		;	15h00:00

fcc		"Example00\0"

CARun00:			ldy		#MonitorStart                   	;   find adr of functions table

ldy     		OffTableRoutines,y              	;   Y = @ of functions table

ldy     		WriteString,y                	;   add offset to point WriteString

ldx		#STRExample00		; string to display

jsr     		0,y

rts

STRExample00		fcc     "Application 00 started\015\012\0"

;	----------------------------------------------------------------------------------------------------

CA_Next01:		fdb		$0000

fdb		CA_Run01

fdb		$0000

fdb		%0100001001110110		;	2013/03/22

fdb		%0111100111100000		;	15h15:00

fcc		"Example01\0"

CA_Run01:		ldy		#MonitorStart

ldy     		OffTableRoutines,y

ldy     		WriteString,y                	;  add offset to point WriteString

ldx		#STRExample01

jsr     		0,y

rts

STRExample01		fcc     "Application 01 started\015\012\0"

## Skeleton of Automatic Expansion ROM

RomCartidgeStart		equ		$8000

MonitorStart		equ		$E000

;	----------------------------------------------------------------------------------------------------

;	Offsets in ROM header

CPUCode			equ		2

Version			equ		6

Date			equ		8

OffTableRoutines		equ		12

;	----------------------------------------------------------------------------------------------------

;	Offsets of subroutines in functions's Monitor

PutChar			equ		0

PutHexChar		equ		PutChar+2

GetChar			equ		PutHexChar+2

GetCharUntil		equ		GetChar+2

WriteHexByte		equ		GetCharUntil+2

WriteBinByte		equ		WriteHexByte+2

WriteString		equ		WriteBinByte+2

ReadString			equ		WriteString+2

ReadHexFromString		equ		ReadString+2

;	----------------------------------------------------------------------------------------------------

org		RomCartidgeStart

fcc		"AT"			;Automatic Cartridge Header

;	----------------------------------------------------------------------------------------------------

Startup:			pshs	x				;save return address

Ldy	#MonitorStart               	   	;find adr of functions table

ldy     	OffTableRoutines,y             		;Y = @ of functions table

ldy     	WriteString,y                		;add offset to point WriteString

ldx	#STRExample00			;string to display

jsr    	 0,y

puls	x				;restore return address

jmp	0,x				;return to the monitor

STRExample00		fcc     "Automatic Cartridge started\015\012\0"

## Skeleton of Diagnostic Expansion ROM

RomCartidgeStart		equ		$8000

MonitorStart		equ		$E000

;	----------------------------------------------------------------------------------------------------

;	Offsets in ROM header

CPUCode			equ		2

Version			equ		6

Date			equ		8

OffTableRoutines		equ		12

;	----------------------------------------------------------------------------------------------------

;	Offsets of subroutines in functions's Monitor

PutChar			equ		0

PutHexChar		equ		PutChar+2

GetChar			equ		PutHexChar+2

GetCharUntil		equ		GetChar+2

WriteHexByte		equ		GetCharUntil+2

WriteBinByte		equ		WriteHexByte+2

WriteString		equ		WriteBinByte+2

ReadString			equ		WriteString+2

ReadHexFromString		equ		ReadString+2

;	----------------------------------------------------------------------------------------------------

org		RomCartidgeStart

fcc		"DG"			;Diagnostic Cartridge Header

;	----------------------------------------------------------------------------------------------------

Startup:			pshs	x				;save return address

ldy	#MonitorStart                   		;   find adr of functions table

ldy     	OffTableRoutines,y              		;   Y = @ of functions table

ldy     	WriteString,y                		;   add offset to point WriteString

ldx	#STRExample00			;string to display

jsr    	 0,y

puls	x				;restore return address

jmp	0,x				;return to the monitor

STRExample00		fcc     "Diagnostic Cartridge started\015\012\0"

;	----------------------------------------------------------------------------------------------------				
Annex: Hardware screenshots

Annex: Monitor Screenshots

Main screen of the Monitor

Tiny Basic running

Dump command at $E000 for 16 values

Some command executed